# Ark-1 v2 — Resilience Buffers (integrated spec)

| Component | Function | Capacity (target) | Placement & interfaces |
| --- | --- | --- | --- |
| A. Food Buffer | Ride out long logistics gaps | 90 t dry, energy-dense (≈1 kg / person / day × 500 × 180 days) | Shield-ring lockers (outer radius) so the mass doubles as radiation shielding; palletized ISO-mini totes; humidity ≤40%, ≤15 °C; RFID lot tracking; 6-month rotation plan (10% churn / month). |
| B. Bioprocess Buffer | Preserve biological IP to “reboot” the loop | Cryo bank: BSFL eggs, algal strains, soil microbiome master mixes; dual-redundant batches | Utility Deck (Layer-0), two separated fire-rated vaults; −80 °C freezers + LN₂ standby; power 8–15 kW steady, 30 kWh UPS each; quarterly viability assays; barcode + chain-of-custody SOP. |
| C. Backup O₂/CO₂ | Bridge O₂ until Sabatier/electrolysis are back up | 30-day air buffer (LOX + chemical scrubbers). Design target: ~15 t total (tight but feasible with mixed media: LOX + solid amine canisters + emergency O₂ candles). | Axial service bay near life-support; LOX in 2× independent dewars; valve trees fail-closed; scrubber racks quick-swap; plumbed to common manifold with check valves. |

Notes on C (air buffer) for realism:

- Crew O₂ demand ≈ 0.84 kg/person/day ⇒ ~12.6 t O₂ for 30 days/500 crew.

- CO₂ scrubbing: plan on solid amines / regenerative beds as primary; carry LiOH or O₂ candles only for contingency. Keeping the total at ~15 t is possible if most CO₂ handling is regenerative and candles are a last resort.

# ABIC Annex A — add Resilience Requirements (drop-in text)

A.7 Resilience Buffers (all ABIC Nodes handling crew ≥100):

- Food Buffer. Nodes shall maintain a non-perishable reserve ≥ 180 crew-days per person (mass basis), with FIFO rotation ≥ 10%/month, humidity control, and batch traceability.

- Bioprocess Cryo Bank. Nodes operating closed-loop life support shall keep dual-vault cryogenic reserves of critical biostock (insect, algal, and microbiome master mixes), with independent power and thermal protection, validated quarterly.

- Backup Atmosphere. Nodes shall provision a minimum 30-day O₂/CO₂ bridge: (i) independent LOX storage and (ii) regenerative CO₂ removal, plus contingency solids sized for 72 h at full crew load.

- Separation & Fire Safety. Buffers must be physically separated from the main habitat by two fire-rated boundaries and tied into emergency power/controls per Annex A.3.

# Throughput & logistics (ABIC lanes / MGMS)

- Mass to stage: 90 t (food) + 15 t (air system) ≈ 105 t one-time; cryo bank ≈ <5 t including vaults/UPS (spread across spares).

- Bulk lane (Ark-class 16 MW depot): ~14 t/day effective → ~7.5 days to stage 105 t once corridors are live.

- Priority lane isn’t needed for these items; keep it for time-critical spares.

- MGMS fit: palletized totes (food) and modular dewars/crates (air/cryo) suit 1–5 t can sizes; stow as shield-ring ballast on arrival.

# Ops & maintenance hooks

- Food: monthly QA (water activity, temp log review); replace 10–15% per month to keep rolling freshness.

- Cryo: dual power (bus A/B), LN₂ reserve ≥72 h, quarterly vial viability spot-checks and reseeding.

- Air buffer: annual pressure tests, quarterly valve exercise; keep candles sealed and dated; simulate a 24 h switch-over drill each quarter.